# Day 3 — Introduction to IP Addressing

## 1. Day 3 Overview

Today we move from “How devices communicate” to “How devices are identified and how networks are divided.”

In Day 2, you learned that the Internet layer uses IP for addressing and routing.

Today you will learn:

- What an IP address is

- IPv4 structure

- IPv6 structure

- Public vs private IP addresses

- Subnet masks

- CIDR notation

- Network address and broadcast address

- Hosts and usable IP addresses

- Basic subnetting

- How to configure a static IP

- How to verify connectivity using ping

- How to solve subnetting problems

# Part A — Theory

# 2. What Is an IP Address?

An IP address (Internet Protocol address) is a logical address assigned to a network interface so that devices can communicate over an IP network.

Think of an IP address like a postal address.

For example:

192.168.1.10

This tells the network where a device is located within an IP addressing structure.

Suppose you have:

Laptop       192.168.1.10

Phone        192.168.1.20

Printer      192.168.1.30

Router       192.168.1.1

All of these devices could belong to the same local network.

Router

192.168.1.1

|

┌───────┼────────┐

│       │        │

Laptop    Phone   Printer

.1.10       .1.20    .1.30

The IP address allows the network stack to identify the source and destination of IP traffic.

# 3. Why Do We Need IP Addresses?

Imagine an office containing 100 computers.

If a computer wants to send data to another computer, the network needs to know:

Who is sending?

Where is the destination?

Which network does the destination belong to?

How should the packet get there?

IP addressing provides the logical addressing needed for this communication.

For example:

Source:

192.168.1.10

Destination:

192.168.1.50

The packet is essentially saying:

Send this IP packet from 192.168.1.10 to 192.168.1.50.

# 4. IPv4

IPv4 is the older and still extremely widespread version of IP.

An IPv4 address is 32 bits long.

It is normally written as four decimal numbers separated by dots.

Example:

192.168.1.10

Each section is called an octet.

192 . 168 . 1 . 10

│     │    │    │

Octet Octet Octet Octet

Each octet contains 8 bits.

Therefore:

8 + 8 + 8 + 8 = 32 bits

# 5. IPv4 in Binary

Computers ultimately work with binary.

For example:

192.168.1.10

can be represented as:

11000000.10101000.00000001.00001010

Each octet contains 8 bits.

The possible values of an octet are:

00000000 = 0

11111111 = 255

Therefore an IPv4 octet can have values from:

0 → 255

# 6. IPv4 Address Structure

An IPv4 address has two logical parts:

Network portion + Host portion

For example:

192.168.1.10/24

The /24 tells us how many bits belong to the network portion.

Network                 Host

<------------------>    <---->

192.168.1             . 10

More accurately:

11111111.11111111.11111111.00000000

<--------- 24 --------> <--- 8 --->

Network              Host

# 7. What Is a Subnet Mask?

A subnet mask tells the device which portion of an IPv4 address represents the network and which portion represents the host.

For example:

IP:

192.168.1.10

Subnet mask:

255.255.255.0

Binary:

IP:

11000000.10101000.00000001.00001010

Mask:

11111111.11111111.11111111.00000000

The 1s represent the network portion.

The 0s represent the host portion.

So:

Network = 192.168.1

Host    = 10