# Part A — IPv4 & CIDR

## 8. CIDR Notation

CIDR stands for Classless Inter-Domain Routing.

Instead of writing an IP address together with its subnet mask separately:

IP Address:    192.168.1.10

Subnet Mask:   255.255.255.0

we can write:

192.168.1.10/24

Here:

- 192.168.1.10 = IP address

- /24 = CIDR prefix length

- /24 tells us that the first 24 bits are network bits

- Remaining 32 - 24 = 8 bits are host bits

### IPv4 always has 32 bits

An IPv4 address has 4 octets:

192 . 168 . 1 . 10

Each octet = 8 bits.

Therefore:

4 × 8 = 32 bits

For /24:

Network bits = 24

Host bits    = 32 - 24 = 8

Visually:

11111111.11111111.11111111.00000000

Network       |     Host

24 bits          8 bits

# How Do We Calculate the Subnet Mask?

This is one of the most important things to understand.

Each octet has 8 bits.

The binary values are:

128  64  32  16  8  4  2  1

For example, /26 means:

26 network bits

The first 24 bits give:

255.255.255

We need 2 more network bits in the last octet:

11000000

Calculate:

128 + 64 = 192

Therefore:

/26 = 255.255.255.192

### Important CIDR table

| CIDR | Subnet Mask | Network Bits | Host Bits |
| --- | --- | --- | --- |
| /8 | 255.0.0.0 | 8 | 24 |
| /16 | 255.255.0.0 | 16 | 16 |
| /24 | 255.255.255.0 | 24 | 8 |
| /25 | 255.255.255.128 | 25 | 7 |
| /26 | 255.255.255.192 | 26 | 6 |
| /27 | 255.255.255.224 | 27 | 5 |
| /28 | 255.255.255.240 | 28 | 4 |
| /29 | 255.255.255.248 | 29 | 3 |
| /30 | 255.255.255.252 | 30 | 2 |

### Easy way to remember

From /24 onward:

/24 → 255.255.255.0

/25 → 255.255.255.128

/26 → 255.255.255.192

/27 → 255.255.255.224

/28 → 255.255.255.240

/29 → 255.255.255.248

/30 → 255.255.255.252

# 9. What Is a Network?

Consider:

192.168.1.10/24

We know:

/24 → 24 network bits

Therefore:

Network bits = 24

Host bits = 8

The first three octets are the network portion:

192.168.1 | 10

-----------   --

Network    Host

So the network is:

192.168.1.0/24

The hosts can be:

192.168.1.1

192.168.1.2

192.168.1.3

...

192.168.1.254

But there are two special addresses:

192.168.1.0   → Network address

192.168.1.255 → Broadcast address

Therefore, normal usable hosts are:

192.168.1.1 → 192.168.1.254

# 10. Network Address

The network address identifies the subnet itself.

For:

192.168.1.10/24

the subnet is:

192.168.1.0/24

Therefore:

Network Address = 192.168.1.0

You normally don't assign this address to an ordinary host.

### How do we calculate it?

The easiest method is:

Set all host bits to 0.

For /24:

192.168.1.10

The last 8 bits are host bits.

Set them to zero:

192.168.1.0

Therefore:

Network = 192.168.1.0

# 11. Broadcast Address

The broadcast address is the last address in the subnet and is used to communicate with all hosts on that IPv4 subnet.

For:

192.168.1.0/24

the complete range is:

192.168.1.0 → 192.168.1.255

Therefore:

Network Address = 192.168.1.0

Broadcast       = 192.168.1.255

And usable hosts:

192.168.1.1 → 192.168.1.254

So remember:

Network → First address

Hosts   → Middle addresses

Broadcast → Last address

# 12. How Many Hosts Can a Subnet Have?

This is a very important formula.

### Formula

Usable Hosts = 2^h - 2

where:

h = number of host bits

Why -2?

Because traditionally:

1 address = Network address

1 address = Broadcast address

are not assigned to ordinary hosts.

## Example: /24

First calculate host bits:

Host bits = 32 - 24

= 8

Then:

Total addresses = 2^8

= 256

Usable:

256 - 2

= 254

Therefore:

/24 = 256 total addresses

/24 = 254 usable hosts

## Example: /26

Host bits = 32 - 26

= 6

Total:

2^6 = 64

Usable:

64 - 2 = 62

Therefore:

/26 = 64 total addresses

/26 = 62 usable hosts

# 13. Important Subnet Table

You should gradually memorize this table:

| CIDR | Host Bits | Total Addresses | Usable Hosts |
| --- | --- | --- | --- |
| /24 | 8 | 256 | 254 |
| /25 | 7 | 128 | 126 |
| /26 | 6 | 64 | 62 |
| /27 | 5 | 32 | 30 |
| /28 | 4 | 16 | 14 |
| /29 | 3 | 8 | 6 |
| /30 | 2 | 4 | 2 |

Notice the pattern:

/24 → 254 hosts

/25 → 126 hosts

/26 → 62 hosts

/27 → 30 hosts

/28 → 14 hosts

/29 → 6 hosts

/30 → 2 hosts

### Why does the number decrease?

When CIDR increases:

/24 → /25 → /26 → /27

you are taking bits away from the host portion.

So there are fewer host addresses.

# 14. What Is Subnetting?

Subnetting means dividing one larger network into smaller networks.

Suppose you have:

192.168.1.0/24

This gives:

256 total addresses

254 usable hosts

But suppose your company has:

HR       → 30 computers

Finance  → 30 computers

IT       → 50 computers

Guest    → 20 computers

Instead of putting everyone into one /24 network, you can divide it into smaller networks.

For example, using /26:

192.168.1.0/26

192.168.1.64/26

192.168.1.128/26

192.168.1.192/26

Each /26 provides:

64 total addresses

62 usable hosts

So four /26 networks can be created from one /24.

# 15. Understanding /26 Properly

Let's calculate:

192.168.1.0/26

### Step 1 — Find host bits

32 - 26 = 6

So:

Host bits = 6

### Step 2 — Find total addresses

2^6 = 64

So each subnet has:

64 addresses

### Step 3 — Find usable hosts

64 - 2 = 62

### Step 4 — Find subnet mask

/26:

255.255.255.192

### Step 5 — Find block size

Use:

Block Size = 256 - subnet mask value

The interesting octet is the last octet:

256 - 192 = 64

Therefore, the subnets increase by 64:

0

64

128

192

So the ranges are:

Subnet 1:

192.168.1.0   → 192.168.1.63

Subnet 2:

192.168.1.64  → 192.168.1.127

Subnet 3:

192.168.1.128 → 192.168.1.191

Subnet 4:

192.168.1.192 → 192.168.1.255

## Let's analyze the first /26

192.168.1.0/26

Range:

192.168.1.0 → 192.168.1.63

Therefore:

Network Address = 192.168.1.0

First Host      = 192.168.1.1

Last Host       = 192.168.1.62

Broadcast       = 192.168.1.63

## Analyze the second /26

192.168.1.64/26

Range:

192.168.1.64 → 192.168.1.127

Therefore:

Network Address = 192.168.1.64

First Host      = 192.168.1.65

Last Host       = 192.168.1.126

Broadcast       = 192.168.1.127

This pattern is extremely useful.

# The Most Important CIDR Calculation Method

If you are given something like:

192.168.10.75/26

and asked to find:

- subnet mask

- network address

- broadcast address

- first host

- last host

- number of hosts

use these steps every time.

### Step 1 — Find host bits

32 - 26 = 6

### Step 2 — Find total addresses

2^6 = 64

### Step 3 — Find usable hosts

64 - 2 = 62

### Step 4 — Find subnet mask

/26 = 255.255.255.192

### Step 5 — Find block size

256 - 192 = 64

### Step 6 — Find which range contains 75

Ranges are:

0–63

64–127

128–191

192–255

75 falls into:

64–127

Therefore:

Network Address = 192.168.10.64

Broadcast       = 192.168.10.127

### Step 7 — Find usable hosts

First Host = 192.168.10.65

Last Host  = 192.168.10.126

### Final answer

IP:              192.168.10.75/26

Subnet Mask:     255.255.255.192

Network:         192.168.10.64

First Host:      192.168.10.65

Last Host:       192.168.10.126

Broadcast:       192.168.10.127

Total Addresses: 64

Usable Hosts:    62

# 16. Real-World Office Example

Suppose an organization has:

Company Network:

192.168.10.0/24

They have four departments:

HR

Finance

IT

Guest Wi-Fi

We can divide the /24 into four /26 subnets.

### HR

192.168.10.0/26

Network:   192.168.10.0

Hosts:     .1 – .62

Broadcast: .63

### Finance

192.168.10.64/26

Network:   192.168.10.64

Hosts:     .65 – .126

Broadcast: .127

### IT

192.168.10.128/26

Network:   192.168.10.128

Hosts:     .129 – .190

Broadcast: .191

### Guest

192.168.10.192/26

Network:   192.168.10.192

Hosts:     .193 – .254

Broadcast: .255

Each subnet has:

64 total addresses

62 usable hosts

This allows departments to be separated logically. In a real network, VLANs, routers/L3 switches, ACLs, and firewalls can then control communication between those subnets.

# 17. IPv4 Private Addresses

Some IPv4 ranges are reserved for private networks.

There are three major private ranges:

| Private Range | CIDR |
| --- | --- |
| 10.0.0.0 – 10.255.255.255 | /8 |
| 172.16.0.0 – 172.31.255.255 | /12 |
| 192.168.0.0 – 192.168.255.255 | /16 |

Examples:

10.0.0.5

10.20.30.40

172.16.10.20

172.20.5.10

192.168.1.50

192.168.100.25

You commonly see these inside:

- home networks

- offices

- laboratories

- virtual machines

- cloud/private networks

### Important

Don't think:

"Every 172.x.x.x address is private."

That's wrong.

Only:

172.16.0.0 → 172.31.255.255

is the private 172 range.

Similarly, only:

192.168.x.x

is private—not every 192.x.x.x.

# 18. Public vs Private IP

## Private IP

Used within private networks.

Example:

192.168.1.10

Your home router might give your laptop:

192.168.1.10

That address is private.

## Public IP

A public IP is globally routable on the Internet, subject to routing and filtering.

Your Internet connection may have a public address assigned by your ISP.

### One important correction to your notes

You have:

203.0.113.x

as an example of a public IP.

Technically, 203.0.113.0/24 is TEST-NET-3, reserved specifically for documentation and examples. So it should not be treated as a real public IP assignment.

For learning, use:

192.168.1.10 → private example

and understand that a genuinely Internet-routable public IP comes from the public address space allocated by the relevant Internet registries/providers.