# Day 3 — Introduction to IP Addressing

## 1. Day 3 Overview

Today we move from “How devices communicate” to “How devices are identified and how networks are divided.”

In Day 2, you learned that the Internet layer uses IP for addressing and routing.

Today you will learn:

- What an IP address is

- IPv4 structure

- IPv6 structure

- Public vs private IP addresses

- Subnet masks

- CIDR notation

- Network address and broadcast address

- Hosts and usable IP addresses

- Basic subnetting

- How to configure a static IP

- How to verify connectivity using ping

- How to solve subnetting problems

# Part A — Theory

# 2. What Is an IP Address?

An IP address (Internet Protocol address) is a logical address assigned to a network interface so that devices can communicate over an IP network.

Think of an IP address like a postal address.

For example:

192.168.1.10

This tells the network where a device is located within an IP addressing structure.

Suppose you have:

Laptop       192.168.1.10

Phone        192.168.1.20

Printer      192.168.1.30

Router       192.168.1.1

All of these devices could belong to the same local network.

Router

192.168.1.1

|

┌───────┼────────┐

│       │        │

Laptop    Phone   Printer

.1.10       .1.20    .1.30

The IP address allows the network stack to identify the source and destination of IP traffic.

# 3. Why Do We Need IP Addresses?

Imagine an office containing 100 computers.

If a computer wants to send data to another computer, the network needs to know:

Who is sending?

Where is the destination?

Which network does the destination belong to?

How should the packet get there?

IP addressing provides the logical addressing needed for this communication.

For example:

Source:

192.168.1.10

Destination:

192.168.1.50

The packet is essentially saying:

Send this IP packet from 192.168.1.10 to 192.168.1.50.

# 4. IPv4

IPv4 is the older and still extremely widespread version of IP.

An IPv4 address is 32 bits long.

It is normally written as four decimal numbers separated by dots.

Example:

192.168.1.10

Each section is called an octet.

192 . 168 . 1 . 10

│     │    │    │

Octet Octet Octet Octet

Each octet contains 8 bits.

Therefore:

8 + 8 + 8 + 8 = 32 bits

# 5. IPv4 in Binary

Computers ultimately work with binary.

For example:

192.168.1.10

can be represented as:

11000000.10101000.00000001.00001010

Each octet contains 8 bits.

The possible values of an octet are:

00000000 = 0

11111111 = 255

Therefore an IPv4 octet can have values from:

0 → 255

# 6. IPv4 Address Structure

An IPv4 address has two logical parts:

Network portion + Host portion

For example:

192.168.1.10/24

The /24 tells us how many bits belong to the network portion.

Network                 Host

<------------------>    <---->

192.168.1             . 10

More accurately:

11111111.11111111.11111111.00000000

<--------- 24 --------> <--- 8 --->

Network              Host

# 7. What Is a Subnet Mask?

A subnet mask tells the device which portion of an IPv4 address represents the network and which portion represents the host.

For example:

IP:

192.168.1.10

Subnet mask:

255.255.255.0

Binary:

IP:

11000000.10101000.00000001.00001010

Mask:

11111111.11111111.11111111.00000000

The 1s represent the network portion.

The 0s represent the host portion.

So:

Network = 192.168.1

Host    = 10

# 8. CIDR Notation

Instead of writing:

192.168.1.10

255.255.255.0

we commonly write:

192.168.1.10/24

The /24 is called CIDR prefix length.

CIDR stands for:

Classless Inter-Domain Routing

It tells us:

The first 24 bits are the network portion.

Common examples:

| CIDR | Subnet Mask | Host Bits |
| --- | --- | --- |
| /8 | 255.0.0.0 | 24 |
| /16 | 255.255.0.0 | 16 |
| /24 | 255.255.255.0 | 8 |
| /25 | 255.255.255.128 | 7 |
| /26 | 255.255.255.192 | 6 |
| /27 | 255.255.255.224 | 5 |
| /28 | 255.255.255.240 | 4 |
| /29 | 255.255.255.248 | 3 |
| /30 | 255.255.255.252 | 2 |

# 9. What Is a Network?

Consider:

192.168.1.10/24

The /24 means:

Network:

192.168.1.0

Host:

10

The network is:

192.168.1.0/24

Possible host addresses are:

192.168.1.1

192.168.1.2

192.168.1.3

...

192.168.1.254

# 10. Network Address

The network address identifies the subnet itself.

For:

192.168.1.10/24

the network address is:

192.168.1.0

You normally don't assign the network address to an ordinary host.

# 11. Broadcast Address

In a traditional IPv4 subnet, the broadcast address is used to reach all hosts on that subnet.

For:

192.168.1.0/24

the broadcast address is:

192.168.1.255

So:

Network address:

192.168.1.0

Usable hosts:

192.168.1.1 → 192.168.1.254

Broadcast:

192.168.1.255

# 12. How Many Hosts Can a Subnet Have?

The basic formula for a traditional IPv4 subnet is:

Usable hosts = 2^h - 2

where h is the number of host bits.

For /24:

32 - 24 = 8 host bits

Therefore:

2^8 = 256 total addresses

Subtract:

1 network address

1 broadcast address

So:

256 - 2 = 254 usable hosts

Therefore:

/24 = 254 usable host addresses.

# 13. Important Subnet Table

Memorize this table gradually.

| CIDR | Total Addresses | Usable Hosts |
| --- | --- | --- |
| /24 | 256 | 254 |
| /25 | 128 | 126 |
| /26 | 64 | 62 |
| /27 | 32 | 30 |
| /28 | 16 | 14 |
| /29 | 8 | 6 |
| /30 | 4 | 2 |

Notice what happens as the prefix increases:

/24 → 254 hosts

/25 → 126 hosts

/26 → 62 hosts

/27 → 30 hosts

/28 → 14 hosts

/29 → 6 hosts

/30 → 2 hosts

You're borrowing bits from the host portion to create smaller networks.

# 14. What Is Subnetting?

Subnetting means dividing one larger network into smaller networks.

Imagine you receive:

192.168.1.0/24

You have 254 usable host addresses.

But suppose your company has three departments:

HR       → 30 computers

Finance  → 30 computers

IT       → 50 computers

Instead of putting everything into one large network, you can divide the network into smaller subnets.

For example:

192.168.1.0/26

192.168.1.64/26

192.168.1.128/26

192.168.1.192/26

Each /26 subnet has:

64 total addresses

62 usable hosts

# 15. Understanding /26

Let's understand this carefully.

A /26 has:

32 - 26 = 6 host bits

Therefore:

2^6 = 64 addresses

Subnet mask:

255.255.255.192

The subnet ranges are:

192.168.1.0   → 192.168.1.63

192.168.1.64  → 192.168.1.127

192.168.1.128 → 192.168.1.191

192.168.1.192 → 192.168.1.255

For the first subnet:

Network:

192.168.1.0

Usable:

192.168.1.1

through

192.168.1.62

Broadcast:

192.168.1.63

# 16. Real-World Example: Office Network

Imagine a small company has:

HR

Finance

IT

Guest Wi-Fi

The network administrator could design:

Company Network

192.168.10.0/24

Then divide it:

HR:

192.168.10.0/26

Finance:

192.168.10.64/26

IT:

192.168.10.128/26

Guest:

192.168.10.192/26

Now each department has its own subnet.

This makes network management and security easier, especially when combined with routing rules, VLANs, and firewalls.

# 17. IPv4 Private Addresses

Some IPv4 ranges are reserved for private networks.

The three major private ranges are:

10.0.0.0/8

172.16.0.0/12

192.168.0.0/16

Examples:

10.0.0.5

172.16.10.20

192.168.1.50

You commonly see addresses such as:

192.168.1.x

192.168.0.x

10.0.0.x

inside homes, offices, labs, etc.

These addresses are not directly routable across the public Internet in the same way public addresses are.

# 18. Public vs Private IP

### Private IP

Used inside private networks.

Example:

192.168.1.10

### Public IP

Used for Internet-facing communication and allocated through Internet addressing systems.

Example:

203.0.113.x

The second range above is actually reserved for documentation, so don't treat it as a real public assignment; real public addresses are allocated to organizations/providers.

# Part B — IPv6

# 19. Why Was IPv6 Introduced?

IPv4 has only:

32 bits

That gives approximately:

4.3 billion

IPv4 addresses.

With billions of Internet-connected devices, address availability became a major concern.

IPv6 uses:

128 bits

This provides an enormously larger address space.

# 20. IPv6 Structure

An IPv6 address is written using hexadecimal.

Example:

2001:db8:1234:5678:abcd:ef01:2345:6789

It contains eight groups.

Each group contains four hexadecimal digits.

Example:

2001

db8

1234

5678

abcd

ef01

2345

6789

Each group represents 16 bits.

Therefore:

8 × 16 = 128 bits

# 21. IPv4 vs IPv6

| Feature | IPv4 | IPv6 |
| --- | --- | --- |
| Address size | 32-bit | 128-bit |
| Format | Decimal | Hexadecimal |
| Example | 192.168.1.10 | 2001:db8::10 |
| Address space | ~4.3 billion | Enormous |
| Broadcast | Supported | No traditional broadcast |
| Address configuration | DHCP/static etc. | SLAAC, DHCPv6, static etc. |
| Header | Variable length | Fixed base header |
| NAT | Very common | Less fundamental to address conservation |

IPv6 is not simply "IPv4 with more addresses"; it also introduces changes to addressing and protocol behavior.

# 22. IPv6 Address Simplification

IPv6 addresses can be shortened.

For example:

2001:0db8:0000:0000:0000:0000:0000:0010

can be written as:

2001:db8::10

Rules allow leading zeroes in groups to be removed and a consecutive sequence of all-zero groups to be compressed with ::.

You should understand this because IPv6 addresses look complicated at first, but they follow consistent rules.

# Part C — Practical Lab

# 23. Static IP Address Practical

Now let's configure a simple network.

Suppose you have two computers:

PC-A

PC-B

Connect them to the same network.

We'll use:

Network:

192.168.10.0/24

Assign:

PC-A:

192.168.10.10

255.255.255.0

PC-B:

192.168.10.20

255.255.255.0

Because both are in:

192.168.10.0/24

they should be able to communicate directly if there is no firewall or other configuration preventing it.

# 24. Configure Static IP on Windows

Open:

Settings

→ Network & Internet

→ Advanced network settings

→ More network adapter options

Find your network adapter.

Right-click:

Properties

Select:

Internet Protocol Version 4 (TCP/IPv4)

Choose:

Use the following IP address

For PC-A:

IP address:

192.168.10.10

Subnet mask:

255.255.255.0

Default gateway:

leave empty for an isolated two-PC lab

For PC-B:

IP address:

192.168.10.20

Subnet mask:

255.255.255.0

Default gateway:

leave empty for an isolated two-PC lab

DNS is also unnecessary for a simple IP connectivity test.

# 25. Verify the Configuration

On Windows:

ipconfig

You should see something similar to:

IPv4 Address:

192.168.10.10

Subnet Mask:

255.255.255.0

On Linux:

ip addr

You can also inspect routes using:

ip route

# 26. Test Connectivity With Ping

From PC-A:

ping 192.168.10.20

If connectivity is working, you'll receive replies similar to:

Reply from 192.168.10.20:

bytes=32 time<1ms TTL=128

The exact output depends on the operating system and network.

This proves that PC-A can reach PC-B at the IP layer.

# 27. What Happens When You Ping?

This is important.

Suppose:

PC-A:

192.168.10.10

PC-B:

192.168.10.20

You run:

ping 192.168.10.20

The ping utility uses ICMP Echo Request messages.

Conceptually:

PC-A                         PC-B

192.168.10.10               192.168.10.20

ICMP Echo Request

-------------------->

ICMP Echo Reply

<--------------------

ICMP operates alongside IP at the Internet layer.

# 28. Observe Ping in Wireshark

This makes today's theory much more practical.

Start Wireshark.

Start capture.

Run:

ping 192.168.10.20

Then filter:

icmp

You should see:

Echo (ping) request

Echo (ping) reply

Click the packet.

You should see something conceptually like:

Frame

└── Ethernet II

└── Internet Protocol Version 4

└── Internet Control Message Protocol

Now connect that to the TCP/IP model:

Ethernet

↓

Network Access

IPv4

↓

Internet

ICMP

↓

Internet

Notice that there is no TCP or UDP header in an ICMP ping packet.

That's an important practical observation.

# 29. Subnetting Exercise 1

### Question

Given:

192.168.1.0/24

How many usable hosts are available?

### Step 1

Find host bits:

32 - 24 = 8

### Step 2

Calculate total addresses:

2^8 = 256

### Step 3

Remove network and broadcast:

256 - 2 = 254

### Answer

254 usable hosts

# 30. Subnetting Exercise 2

Given:

192.168.1.0/26

Find:

- Subnet mask

- Number of subnets if starting from /24

- Total addresses per subnet

- Usable hosts

- Network addresses

### Step 1 — Mask

/26:

255.255.255.192

### Step 2 — Host bits

32 - 26 = 6

### Step 3 — Total addresses

2^6 = 64

### Step 4 — Usable hosts

64 - 2 = 62

### Step 5 — Subnets within /24

You borrowed:

26 - 24 = 2 bits

Therefore:

2^2 = 4 subnets

They are:

192.168.1.0/26

192.168.1.64/26

192.168.1.128/26

192.168.1.192/26

# 31. Complete /26 Table

| Subnet | Network | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- |
| 1 | 192.168.1.0 | .1 | .62 | .63 |
| 2 | 192.168.1.64 | .65 | .126 | .127 |
| 3 | 192.168.1.128 | .129 | .190 | .191 |
| 4 | 192.168.1.192 | .193 | .254 | .255 |

Each subnet:

64 total

62 usable

# 32. Subnetting Exercise 3 — Find the Subnet

Given:

192.168.10.75/26

Which subnet does it belong to?

The /26 block size is:

256 - 192 = 64

Therefore the ranges are:

0–63

64–127

128–191

192–255

75 falls inside:

64–127

Therefore:

Network:

192.168.10.64/26

First usable:

192.168.10.65

Last usable:

192.168.10.126

Broadcast:

192.168.10.127

# 33. Subnetting Exercise 4

Given:

10.10.5.130/27

Find:

- Subnet mask

- Network address

- First usable address

- Last usable address

- Broadcast address

- Number of usable hosts

### Answer

/27 means:

Subnet mask:

255.255.255.224

Block size:

256 - 224 = 32

Subnet ranges:

0–31

32–63

64–95

96–127

128–159

160–191

192–223

224–255

130 falls within:

128–159

Therefore:

Network:

10.10.5.128

First usable:

10.10.5.129

Last usable:

10.10.5.158

Broadcast:

10.10.5.159

Host bits:

32 - 27 = 5

Total addresses:

2^5 = 32

Usable:

32 - 2 = 30

### Final answer

Network:       10.10.5.128/27

First host:    10.10.5.129

Last host:     10.10.5.158

Broadcast:     10.10.5.159

Usable hosts:  30

# 34. Real-World Subnetting Scenario

Imagine a company receives:

192.168.50.0/24

It has four departments:

| Department | Required Hosts |
| --- | --- |
| IT | 50 |
| HR | 20 |
| Finance | 20 |
| Management | 10 |

A basic fixed-size approach could use /26 for each department:

IT:

192.168.50.0/26

HR:

192.168.50.64/26

Finance:

192.168.50.128/26

Management:

192.168.50.192/26

Each gives 62 usable hosts.

This works, but it wastes addresses for departments needing only 10 or 20 hosts.

A more efficient real-world design would use VLSM (Variable Length Subnet Masking) so that different departments receive different subnet sizes.

That is a topic worth learning after you are comfortable with basic subnetting.

# 35. Day 3 Troubleshooting Scenario

Suppose you configure:

PC-A:

192.168.10.10/24

PC-B:

192.168.20.20/24

You run:

ping 192.168.20.20

but there is no router connecting the networks.

Why?

Because:

PC-A → 192.168.10.0/24

PC-B → 192.168.20.0/24

They are in different IP networks.

A router or Layer-3 device is needed to route between those networks.

Compare that with:

PC-A:

192.168.10.10/24

PC-B:

192.168.10.20/24

They belong to the same subnet and can communicate directly at the local network level, assuming the physical/network configuration and firewall permit it.

# 36. Day 3 Practical Checklist

### Lab 1 — Static IP

- Configure PC-A with 192.168.10.10/24

- Configure PC-B with 192.168.10.20/24

- Verify with ipconfig or ip addr

- Ping PC-B from PC-A

- Ping PC-A from PC-B

### Lab 2 — Wireshark

- Start Wireshark

- Capture traffic

- Run ping

- Filter using icmp

- Identify source IP

- Identify destination IP

- Identify ICMP Echo Request

- Identify ICMP Echo Reply

- Expand Ethernet and IPv4 headers

### Lab 3 — Subnetting

Practice finding:

- CIDR prefix

- Subnet mask

- Network address

- First usable address

- Last usable address

- Broadcast address

- Number of usable hosts

- Number of subnets

# 37. What You Should Be Able to Explain After Day 3

You should be comfortable answering questions like:

### Beginner

What is an IP address?

A logical address used to identify a network interface and facilitate communication over an IP network.

What is IPv4?

A 32-bit IP addressing system represented using four decimal octets.

What is IPv6?

A 128-bit successor to IPv4 that provides a vastly larger address space and other protocol improvements.

What is a subnet mask?

A value that identifies which bits of an IPv4 address represent the network and which represent the host.

What does /24 mean?

The first 24 bits are the network prefix, leaving 8 host bits.

### Practical

If you see:

192.168.100.75/26

you should immediately be able to calculate:

Mask:       255.255.255.192

Network:    192.168.100.64

First host: 192.168.100.65

Last host:  192.168.100.126

Broadcast:  192.168.100.127

Hosts:      62

### Troubleshooting

If:

192.168.1.10/24

cannot communicate with:

192.168.2.10/24

you should start thinking:

These devices are on different IP networks. Is there a router/default gateway and is routing configured?

That is the mindset you want to develop in Day 3—not just memorizing subnetting formulas, but being able to look at an IP configuration and understand what the network is doing.