# Part A — IPv4 & CIDR

## 8. CIDR Notation

CIDR stands for Classless Inter-Domain Routing.

Instead of writing an IP address together with its subnet mask separately:

IP Address:    192.168.1.10

Subnet Mask:   255.255.255.0

we can write:

192.168.1.10/24

Here:

- 192.168.1.10 = IP address

- /24 = CIDR prefix length

- /24 tells us that the first 24 bits are network bits

- Remaining 32 - 24 = 8 bits are host bits

### IPv4 always has 32 bits

An IPv4 address has 4 octets:

192 . 168 . 1 . 10

Each octet = 8 bits.

Therefore:

4 × 8 = 32 bits

For /24:

Network bits = 24

Host bits    = 32 - 24 = 8

Visually:

11111111.11111111.11111111.00000000

Network       |     Host

24 bits          8 bits

# How Do We Calculate the Subnet Mask?

This is one of the most important things to understand.

Each octet has 8 bits.

The binary values are:

128  64  32  16  8  4  2  1

For example, /26 means:

26 network bits

The first 24 bits give:

255.255.255

We need 2 more network bits in the last octet:

11000000

Calculate:

128 + 64 = 192

Therefore:

/26 = 255.255.255.192

### Important CIDR table

| CIDR | Subnet Mask | Network Bits | Host Bits |
| --- | --- | --- | --- |
| /8 | 255.0.0.0 | 8 | 24 |
| /16 | 255.255.0.0 | 16 | 16 |
| /24 | 255.255.255.0 | 24 | 8 |
| /25 | 255.255.255.128 | 25 | 7 |
| /26 | 255.255.255.192 | 26 | 6 |
| /27 | 255.255.255.224 | 27 | 5 |
| /28 | 255.255.255.240 | 28 | 4 |
| /29 | 255.255.255.248 | 29 | 3 |
| /30 | 255.255.255.252 | 30 | 2 |

### Easy way to remember

From /24 onward:

/24 → 255.255.255.0

/25 → 255.255.255.128

/26 → 255.255.255.192

/27 → 255.255.255.224

/28 → 255.255.255.240

/29 → 255.255.255.248

/30 → 255.255.255.252

# 9. What Is a Network?

Consider:

192.168.1.10/24

We know:

/24 → 24 network bits

Therefore:

Network bits = 24

Host bits = 8

The first three octets are the network portion:

192.168.1 | 10

-----------   --

Network    Host

So the network is:

192.168.1.0/24

The hosts can be:

192.168.1.1

192.168.1.2

192.168.1.3

...

192.168.1.254

But there are two special addresses:

192.168.1.0   → Network address

192.168.1.255 → Broadcast address

Therefore, normal usable hosts are:

192.168.1.1 → 192.168.1.254

# 10. Network Address

The network address identifies the subnet itself.

For:

192.168.1.10/24

the subnet is:

192.168.1.0/24

Therefore:

Network Address = 192.168.1.0

You normally don't assign this address to an ordinary host.

### How do we calculate it?

The easiest method is:

Set all host bits to 0.

For /24:

192.168.1.10

The last 8 bits are host bits.

Set them to zero:

192.168.1.0

Therefore:

Network = 192.168.1.0

# 11. Broadcast Address

The broadcast address is the last address in the subnet and is used to communicate with all hosts on that IPv4 subnet.

For:

192.168.1.0/24

the complete range is:

192.168.1.0 → 192.168.1.255

Therefore:

Network Address = 192.168.1.0

Broadcast       = 192.168.1.255

And usable hosts:

192.168.1.1 → 192.168.1.254

So remember:

Network → First address

Hosts   → Middle addresses

Broadcast → Last address

# 12. How Many Hosts Can a Subnet Have?

This is a very important formula.

### Formula

Usable Hosts = 2^h - 2

where:

h = number of host bits

Why -2?

Because traditionally:

1 address = Network address

1 address = Broadcast address

are not assigned to ordinary hosts.

## Example: /24

First calculate host bits:

Host bits = 32 - 24

= 8

Then:

Total addresses = 2^8

= 256

Usable:

256 - 2

= 254

Therefore:

/24 = 256 total addresses

/24 = 254 usable hosts

## Example: /26

Host bits = 32 - 26

= 6

Total:

2^6 = 64

Usable:

64 - 2 = 62

Therefore:

/26 = 64 total addresses

/26 = 62 usable hosts

# 13. Important Subnet Table

You should gradually memorize this table:

| CIDR | Host Bits | Total Addresses | Usable Hosts |
| --- | --- | --- | --- |
| /24 | 8 | 256 | 254 |
| /25 | 7 | 128 | 126 |
| /26 | 6 | 64 | 62 |
| /27 | 5 | 32 | 30 |
| /28 | 4 | 16 | 14 |
| /29 | 3 | 8 | 6 |
| /30 | 2 | 4 | 2 |

Notice the pattern:

/24 → 254 hosts

/25 → 126 hosts

/26 → 62 hosts

/27 → 30 hosts

/28 → 14 hosts

/29 → 6 hosts

/30 → 2 hosts

### Why does the number decrease?

When CIDR increases:

/24 → /25 → /26 → /27

you are taking bits away from the host portion.

So there are fewer host addresses.

# 14. What Is Subnetting?

Subnetting means dividing one larger network into smaller networks.

Suppose you have:

192.168.1.0/24

This gives:

256 total addresses

254 usable hosts

But suppose your company has:

HR       → 30 computers

Finance  → 30 computers

IT       → 50 computers

Guest    → 20 computers

Instead of putting everyone into one /24 network, you can divide it into smaller networks.

For example, using /26:

192.168.1.0/26

192.168.1.64/26

192.168.1.128/26

192.168.1.192/26

Each /26 provides:

64 total addresses

62 usable hosts

So four /26 networks can be created from one /24.